[image: image1.emf]
Specification Statements FSH Products      


1. High Security Electric Door Strike EcoStrike with Pre-Load Capability FES90M-P
“The electric door strike must be a low current motorized locking device with a pre-load/side-load tolerance of 35kg, and durability tested to 1.5 Mill. full pre-load cycles of at least 35 kg. 
The strike has to accept voltages from 10 to 30VDC, and be site interchangeable, “power to lock” & “power to open”.
The door strike must provide 3 forms of monitoring:

1. Lock Status Sensor  
monitoring the internal locking mechanism 
2. Door Latch Sensor  
monitoring Door latch/Lock tongue engaged within strike keeper
3. Door Position Sensor 
monitoring the door position via built-in Reed Switch
The door strike must be provided with a 5 year warranty and fire tested up to a 4 hour fire resistance rating to both Australian and British Fire Test Standards. The device has to have a holding strength of up to 1370kg. The device also has to have a 10mm lip and be pre-drilled for extension lips.  As a truly Green Product, the EcoStrike FES90M-P has to provide a “Sleep-Mode-Function” drawing no more than 15mA@12VDC”

Product features

· 5 year warranty

· Simple power to lock/power to release interchange
· Strike Lip 10mm, pre drilled for extension lips

· Mounting kit included

· Superior holding strength of up to 1370kg

· 4 hour fire rated

· Accepts 10 to 30VDC
· Fully monitored including built-in Reed Switch
2. Mortice Lock Face Plate with monitoring magnets compatible to FES90M-P
“The Mortice Lock Face Plate must be equipped with specifically positioned magnets to utilize door position monitoring and work in conjunction with the strikes built-in Reed Switch.
The Mortice Lock Face Plates are designed to replace existing face plates of mechanical  mortice locks. The mechanical mortice lock brand and type has to be specified
Tuesday, 5 November 2013

[image: image1.emf]